I - Configuring sii680ide through UBOOT envvars

===============================================

Currently sii680ide is configurable through the following UBOOT envvars :

- sii0680ide_verbose

Legal values are : set to anything makes the driver verbose at boot time.

Default if unset : driver silent.

- sii0680ide_maxbus

Legal values are :  0 (no IDE interface at all),

1 (only primary IDE interface), and

2 (both IDE interfaces)

Default : if you don't set this envvar, sii0680ide will use both IDE interfaces.

- sii0680ide_timeout

Legal values are between 1 and 30 (seconds). The latter is the recommended

value from the ATA(PI) spec.

Default : if you don't set this envvar, sii0680ide will use a 20 second timeout.

- sii0680ide_conf

Compose 4 chars (prim master, prim slave, sec master, sec slave) using :

0 : nothing

1 : harddisk

2 : dvd/cdrom reader or burner

Default : if you don't set this envvar, sii0680ide will scan the IDE buses and

try to determine the configuration by itself.

- sii0680ide_xfer

Allows to define the xfer mode which will be used for each drive. Compose

4 chars (prim master, prim slave, sec master, sec slave) using :

0 - Automatic PIO (driver will use the best PIO mode

that the drive claims to support)

a - PIO 0  (  3 MB/s, modeid 8)

b - PIO 1  (  5 MB/s, modeid 9)

c - PIO 2  (  8 MB/s, modeid 10)

d - PIO 3  ( 11 MB/s, modeid 11)

e - PIO 4  ( 16 MB/s, modeid 12)

A - UDMA 0 ( 16 MB/s, modeid 64)

B - UDMA 1 ( 25 MB/s, modeid 65)

C - UDMA 2 ( 33 MB/s, modeid 66)

D - UDMA 3 ( 44 MB/s, modeid 67)

E - UDMA 4 ( 66 MB/s, modeid 68)

F - UDMA 5 (100 MB/s, modeid 69)

G - UDMA 6 (133 MB/s, modeid 70)

NB : sii0680ide will fall back to the best mode that the drive claims to support

in case you ask for an unsupported mode.

Default : if you don't set this envvar, sii0680ide will use the best PIO mode

supported by the drive.

- sii0680ide_irq

Compose 4 chars (prim master, prim slave, sec master, sec slave) using :

1 : use IRQs

any other char : don't use IRQs

Default : if you don't set this envvar, sii0680ide WILL use interrupts.

Using interrupts relieves the CPU in all cases (PIO : approx 20%, and UDMA

approx 95%), but it also has an incidence on xfer speed : 10 to 20% lower

for PIO, 10 to 20% faster for UDMA. This is normal (in PIO, it involves 2

exec reschedulings, and in UDMA, it releases the VIA bus).

EXAMPLE : to use only the primary interface, set ide timeout to 5 seconds,

force prim master as harddisk, sec master as cd, nothing for other

drives, force prim master to udma33 and let sii0680ide select the

xfer mode for other drives, enter the following commands at the

UBOOT prompt :

setenv sii0680ide_maxbus 1

setenv sii0680ide_timeout 5

setenv sii0680ide_conf 1020

setenv sii0680ide_xfer C000

saveenv

Then reboot. Don't forget the 'saveenv', as sii0680ide uses nonvolatile.library,

which only accesses the envvars stored in nvram.

The driver supports up to 4 sii0680 PCI boards / chips. For the 2nd, 3rd etc.

chips, add '.2', '.3' etc. to envvar names. I.e., setenv sii0680ide_conf.2 1020

II - Notes

==========

- This driver can work happily aside the other OS4 disk

drivers.

- This configurability feature relies on two kickstart modules from Ross

Vumbaca that MUST be listed in your kicklayout : nonvolatile.library.kmod

50.2+ and nvram.resource.kmod. If those modules are not present, sii0680ide

will revert to default values.

- These envvars HAVE to be saved ('saveenv' after modifying them), because

they are read directly from the nvram.

- To delete an envvar, just define a blank content & then saveenv (ie

'setenv sii0680ide_conf' and then 'saveenv' will delete this envvar).

- Using sii0680ide_conf (properly set !) saves need for timeouts for drive

detection at boot time.

- It looks like each CD/CDR/DVD/DVDR drive reports its own status codes

when a blank media is inserted. Thus, OS4 will report different info

on different systems when a blank CDR is inserted.

- Colin Wenzel reports that the IDE ZIP works.

- When you grab a boot log & send it to me, please use a nondebug kernel.

- If you experience VERY SLOW harddisk writes with FFS, then you probably

have a drive that does not execute properly a FLUSH_CACHE command

(sii0680ide checks that at init time). Thus, sii0680ide disables the

drive's write cache. Thus, your drive really crawls in writing.

This does not show for the same drive with other drivers under other

os'es, because they don't check if the drive can execute a FLUSH_CACHE,

and just leave the write cache ON.

Under Linux or AOS3.x, this can *look* fast & reliable, but this *is* fast

and not reliable.

Some filesystems depend on proper execution of FLUSH_CACHE, when *they*

decide it, not when the drive decides. Thus, those fs'es can appear to be

working, but they are not really reliable. SFS is one of those.

Thus, it was decided (long ago) by the os4 dev team (mainly js & myself ;),

to disable the write cache if the drive cannot properly execute a

FLUSH_CACHE.

The consequence is that under those circumstances, FFS really crawls at

write, but you can move to the latest SFS + its diskcache.library,

which works around this problem while (mostly) staying on the safe side.

Make sure to create your SFS partition this way :

- 500 buffers

- blocksize set to 512

- maxtransfer set to 7FFFFFFF

- mask set to FFFFFFFE

III - Art of adequate IDE cabling

=================================

- Only use 80 wire cables with 40 pin connectors (UDMA) on a1.

- Put them the proper way around : the mobo HAS to be on the end of the

longest segment with the MASTER at the OTHER END of the cable, and

the slave in the middle, if any.

For 40 wire cables (non UDMA), the layout is :

MOBO <=======> MASTER <=== wire 28 cut ===> SLAVE

For 80 wire cables (UDMA), the layout is :

MOBO <===+===> MASTER

|

wire 56 cut

|

v

SLAVE

- JUMP your drives explicitly as MASTER or SLAVE : no cable select.

I've seen that concept fail already (certainly due to faulty drive

firmware), so I just dont trust it. si680ide knows nothing about cable select,

it's a purely hardware concept. It is a way of letting each drive know if

it's master or slave by having its CSEL signal grounded (by MOBO) or not.

The drives pull the CSEL signal up, and the MOBO grounds the CSEL signal,

thus the SLAVE will see a +5V on the CSEL signal while the MASTER will see

a GND. That's how the slave knows he's a slave & the master knows it's a

master. That also applies to PC's.

For a complete reference, refer to ATA specs, www.t13.org, d1532v2r3e.pdf.

- About ROUND CABLES : they are a cosmetic nonsense. The ATA spec states that

the move from 40 to 80 wires intends to reduce diaphony between data lines,

and you more or less kill that with a round cable.

IV - Fine Art of drive partitioning

===================================

A few things you must know :

- You'll get decent performance from any FS only with buffers >= 500

- You'll get decent performance from FFS only with blocksize >= 2048

- You'll get the best performance from SFS only with blocksize = 512

- SFS is way faster than FFS in its default (non-caching) behaviour,

but FFS is said to be safer.

- SFS + its diskcache.library is even faster.

- There is a cache plugin for FFS, and you can also set FFS in

writeback cache mode. Using both things make it waaaaay faster

(almost comparable to SFS). The FFS cache plugin is 'fs_cache_plugin',

you will find it somewhere on your OS4 CD, just 'run fs_cache_plugin DH0:'

for each FFS partition you wish to run with cache.

- The command I use to bench r/w perf is scsispeed, found in the

latest diskspeed archive from Aminet, and also in an OS4 native

form on the OS4 beta server (quick port by myself, dropped the

m68k CPU usage measurement code).

V - KNOWN PROBLEMS

==================

- SBauer's old Mitsumi 4x cd drive banz0ring (this one always reports

2352 sector size for data cd's, should be 2048). Fixed in 51.33, now

we always use 2048 byte sector size for atapi... Unfixed in 51.34,

2048 broke ZIPs.

- SRup's old GoldStar 4x cd drive banz0ring also (this one reports itself

as a harddisk !).

- AdamK's lg crd8522 not working - but this one is known to be all but ata

conformant.

- SSolie's toshiba cd drive failing 55/AA drive detection code.  Fixed by

the force-config uboot envvars.

- Some philips cdrw drives failing to operate properly (tdg).

- My own WDC WD200BB-32CXA0 wont read more than 1 sec in a READ_SECTORS...