MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

Detailed Specifications for the Multi-Spectra Extraction Package

F. Valdes

December 8, 1983

1.  Introduction

The  multi-spectra  extraction  package  (MULTISPEC)  provides  the

basic tools for modeling, cleaning, and extracting spectra  from  images

containing  multiple  aperture  spectra running roughly parallel.  These

tools will generally be combined  in  reduction  script  tasks  but  may

also be used directly for non-standard analysis.

This  design  presents  the requirements and specifications for the

MULTISPEC package.  Details concerning the algorithms  are  given  in  a

separate document, Algorithms for the Multi-Spectra Extraction Package.

2.  Input Data Requirements

The  input  data  for  the MULTISPEC package consists of image files

containing one or more aperture spectra.  The spectra  are  required  to

run   roughly  parallel  to  each  other  and  parallel  to  the  second

digitization  axis.   The  latter  requirement  may  require  a  general

rotation  and  interpolation  image operator.  The images are assumed to

be  corrected  to  linear  relative  intensity.   Thus,  the  steps   of

correcting   digital   detector  images  for  dark  current,  bias,  and

pixel-to-pixel sensitivity variations must  be  performed  before  using

the MULTISPEC tasks.

Because  the  the MULTISPEC package is being developed concurrently

with the IRAF standard image processing tools  this  document  specifies

the   requirements  for  the  preliminary  image  processing  needed  to

prepare digital detector images for the MULTISPEC package.

2.1  Basic Digital Detector Reduction Tasks

The prelimary reduction of multi-spectra  images  uses  CL  scripts

based  on  general  image  operators.   Some  of  the  scripts  are  for

specific instruments or specific reduction  applications  and  some  are

generally   useful  image  processing  tasks.   The  scripts  allow  the

specification  of  many  images  for  which  the  operations   will   be

repetitively applied.

The  following  CL  scripts  are  required  to reduce multi-spectra

images from digital detectors.

debias          multispec_flat          flat_divide

-1-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

debias

The files in a list of  filenames  are  automatically  debiased  and

trimmed.   This  routine  will  be  instrument  specific but used by

other reduction tasks beyond MULTISPEC.

multispec_flat

The files in a list of quartz  multi-spectra  filenames  are  added,

the  result  is  smoothed  along  the dispersion dimension, and then

the original image is divided by the smoothed  image  to  produce  a

flat  field  image.   The  unsmoothed  to smoothed ratio is computed

only  if  the  value  of  the  smoothed  pixel  is  greater  than  a

specified  amount.   Otherwise,  the  ratio  is  set to unity.  This

routine is not instrument specific but is used  only  for  MULTISPEC

reductions.

flat_divide

The  files  in  a  list of filenames are each divided by a specified

flat field image.  This routine is  not  instrument  or  application

specific.

The  required  general image processing programs needed to implement

these scripts are specified below.

(1) A routine to compute the average value from a specified area of  the

image.  Used to determine the average bias value from a bias strip.

(2) A  routine  to  trim  a specified portion of an image.  Used to trim

the bias strip.

(3) Routines to multiply and subtract images by  a  constant.   Used  to

scale  images  such as dark exposures and to remove the average bias

value as obtained by (1) above.

(4) Routines to subtract, add, and  divide  images.   Used  to  subtract

dark  current  and  bias  exposures,  to  add  several  exposures to

increase the signal-to-noise, and to divide by a flat  field  image.

The  divide  routine  must  give the user the option to substitute a

constant or ignore any divisions in which the  denominator  is  less

than a specified value.

(5) A  routine  to  rotate  or  transpose  an  image.  Used to align the

spectra along lines or columns.

(6) A  routine  to  apply  a  filter  to  lines  of  the   image.    For

multi-spectra  images  a  smooth  quartz  is  produced  by  using  a

running  quadratic  filter  along  each  line  of   the   dispersion

dimension.   The  filter  must  be  able  to  recognize  bad  pixels

(specified by a user defined threshold) and  remove  them  from  the

filtering operation.

-2-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

3.  Requirements for the MULTSPEC Package

The MULTISPEC package shall satisfy the following requirements.

(1) The component programs shall be CL callable.

(2) The  programs  shall interact only through image files and MULTISPEC

data files.

(3) It shall be possible to extract spectra without modeling.

(4) The entire image shall be extracted and not limited by  failures  in

the algorithms.

(5) It  shall  be  possible  to  specify specific lines or swaths in the

image on which to operate.

(6) CL scripts shall be provided for the  common  data  sources.   These

scripts will work automatically.

The follow functions shall be provided:

o   Make  an  initial  rough but automated identification of the spectra

locations.

o   Provide for a user identification list for  the  spectra  locations.

This  list  shall  be  of  the  standard  image cursor type to allow

generation of the list with the standard image cursor programs.

o   Determine and correct for a slowly varying background.

o   Reliably and accurately trace spectra in the presence  of  geometric

distortions (pincushion, s, shear, etc.).

o   Extract spectra by one of:

a.  Strips  of  constant width about the located spectra.  The width

may be specified to fractions of  a  pixel  and  the  extraction

will use fractional pixel interpolation.  l

b.  Strips of width proportional to a Gaussian width parameter.

c.  Modeling  to  obtain  estimates  of  the  total luminosity.  The

estimate will be the integral of the model.

d.  Summation   of   the   data   pixel   values   with   fractional

contributions  of  the  pixel  value  to  the  spectra  based on

modeling.

o   An option shall be available to  specify  whether  to  ignore  blank

pixels or use interpolated values.

o   Programs  shall  be  provided to produce data files which can be

-3-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

accessed by one dimensional  spectroscopic  reduction  routines.

At a minimum these formats shall include:

a.  Reduction  to  an  image  file  consisting  of  one line per

extracted spectrum

b.  The  standard  IIDS  format   available   with   the   CYBER

Multi-Aperture Plate programs

3.2  Modeling Requirements

The  modeling  of multi-spectra images, particularly in the case of

blended spectra, shall:

(1) Model blended spectra with  sufficient  reliability  and  robustness

that  a  reasonable  solution is always obtained, though of possibly

limited usefulness.

(2) The modeling shall provide estimates for the  uncertainties  in  the

fitted parameters as a function of position along the spectrum.

(3) Remove  cosmic  rays  and other defective pixels by reference to the

model.

(4) Allow the transfer of a model solution  for  one  image  to  another

image.

(5) Display  numerically and graphically the data, the fitted model, and

the residuals.

4.  Program Specifications

4.1  Basic Programs

The basic programs of the package are general purpose  tools  which

initialize  a  MULTISPEC  data  file  and  perform  a single fundamental

operation on the data in the MULTISPEC data file.   There  is  one  data

file  associated  with  each  image.   The  data file is hidden from the

user and so the user need not be aware  of  the  data  file.   The  data

files  are  referenced  only the image filename specified in the program

parameters.  The data files contain such  information  as  a  processing

history,  the  spectra  positions  and extracted luminosities, the model

parameters (one set for each spectra for each modelled  image  line  (or

swath),  etc.   The  programs  generally are allowed to specify specific

lines, columns, and/or spectra on which to operate.   The  line,  column

and  spectra  specifications  are given as strings which contain numbers

separated by whitespace, commas,  and  the  range  indicator  "-".   The

script  tasks  of  section  4.2  will  combine  these  basic programs to

perform a general multi-spectra extraction.

-4-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

ap_plate            copy_params     find_spectra    convolve

fit_bckgrnd         find_bckgrnd    line_list       model_extrac

model_fit           model_image     model_list      sigma_extract

strip_extract       to_iids         to_image        to_onedspec

ap_plate

The information from an on-line data  file  containing  descriptions

of  all  the aperture plates prepared at Kitt Peak is read to find a

specified  aperture  plate.   The  drilled  aperture  positions  are

correlated  with  the  spectra  in  the  image  to  deduce  relative

wavelength offsets.  The identifications for  the  spectra  as  well

as  other  auxiliary  information  is recorded in the data file.  If

no  image  file  is  specified  then   only   the   aperture   plate

information   is   printed.    This   program   is   used   in   the

MULTIAP_EXTRACT program.  This  program  is  not  essential  to  the

operation of the MULTISPEC package.

Multi-Spectra image  image =

Aperture plate  plate =

(mode = ql)

-5-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

The Background

The  are  two possibilities for dealing with the background.  In the

first  case,  FIT_BCKGRND,  the  background  will   be   fitted   by

polynomials  and  the  coefficients  stored  in  the  MULTISPEC data

file.  These coefficients are then used by  the  other  programs  to

estimate   the  background  at  the  spectra.   The  second  option,

FIND_BCKGRND, generates a background  image  in  which  the  spectra

and  other  selected  areas are set to blank pixels.  Then a general

image interpolator is used fill in the blank pixels with  background

estimates.   The  other  MULTISPEC  programs  will  then access this

background frame.  The background frame image name  will  be  stored

in the MULTISPEC data file and the image header.

fit_bckgrnd

Fit  a  background  in  a  MULTISPEC image by a polynomial using

pixels not near the spectra and in  the  user  specified  swaths

and  columns.   The buffer distance is in pixels and refers to a

minimum distance from the center of any  spectrum  beyond  which

the  background  pixels  are found.  Blank pixels are ignored in

the background fit.  Deviant pixels will be rejected.

Multi-Spectra image  image =

Buffer from spectra  buffer = 12

Polynomial order  order = 3

Lines per swath (lines_per_swath = 32)

Swaths to fit (swaths = 1-1000)

Columns to fit (columns = 1-1000)

Rejection threshold (threshold = 5)

Print general diagnostics (verbose = no)

(mode = ql)

find_bckgrnd

The spectra within a buffer distance  and  specified  areas  are

set  to  blank  pixels  and  the  remaining  pixels  copied to a

background image file.

Multi-Spectra image  image =

Background image  background =

Buffer from spectra  buffer = 12

Lines to ignore (lines = )

Columns to ignore (columns = )

(mode = ql)

convolve

A program will be provided to reduce either the  extracted  spectrum

or the modeled image to a common point-spread function.

-6-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

copy_params

Create  a  MULTISPEC  data  file  for  a new image using appropriate

MULTISPEC parameters from an old image.  The  old  image  must  have

been  processed  to find the spectra using FIND_SPECTRA and possibly

model fit.

Old Multi-Spectra image  old_image =

New Multi-Spectra image  new_image =

(mode = ql)

find_spectra

Initially locate the spectra in a MULTISPEC  image.   The  positions

of  the  spectra  within the range of columns are determined for the

starting line and then the spectra are tracked within the  range  of

lines.   The  minimum  separation  and minimum width would generally

be set for a particular instrument.   If  the  automatic  search  is

not  used  then a list of cursor positions is read from the standard

input.

Multi-Spectra image  image =

Automatic search  auto = yes

Starting line  start_line =

Minimum separation (min_sep = 1)

Minimum width (min_width = 1)

Averaging width (average = 32)

Lines to search (lines = 1-1000)

Columns to search (columns = 1-1000)

Print general diagnostics (verbose = no)

(mode = ql)

line_list

For the  specified  lines  in  the  image  print  the  image  column

number,  data  value  (possibly as a swath average), the model value

at that point (i.e. the sum of  the  model  contributions  from  all

the  spectra),  the  background  value,  and the residual.  Plotting

scripts may be written using this routine to show the quality  of  a

model fit.

Multi-Spectra image  image =

Lines to list (lines = 1-1000)

(mode = ql)

-7-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

model_extract

A  previously  fitted  model  is  used  to extract the spectra total

luminosity by apportioning the data values to spectra in  the  ratio

indicated  by  the model.  If the clean option is specified then the

model is used to detect pixels which deviate from  the  model  by  a

specified  amount.   The  model  value replaces the deviant pixel in

the extraction and, if specified, also in the image file.

Multi-Spectra image  image =

Lines to extract (lines = 1-1000)

Clean spectra (clean = yes)

Cleaning threshold (threshold = 5)

Modify image (modify = yes)

Print general diagnostics (verbose = no)

(mode = ql)

model_fit

A specified model is iteratively fitted to the data in each  of  the

specified  lines  (or  swaths)  until  the  RMS  residual  fails  to

decrease.  The models  are  selected  by  a  string.   The  possible

values are

(null string) - initialize the model

i - fit only the intensity scale

ip - fit the intensity scale and the position

ips1 - fit the intensity scale, position, and one parameter shape

ips2 - fit the intensity scale, position, and two parameter shape

ips3 - fit the intensity scale, position, and three parameter shape

ips4 - fit the intensity scale, position, and four parameter shape

These  models  will  be  combined in a script to search for the best

fit.

The initial shape parameters will generally be set by scripts for  a

particular data reduction.

Multi-Spectra image  image =

Model type  model =

Lines per swath (lines_per_swath = 32)

Swaths to model (swaths = 1-1000)

Initial shape1 (shape1 = .1 )

Initial shape2 (shape2 = 0 )

Initial shape3 (shape3 = 0 )

Initial shape4 (shape4 = 5 )

Print general diagnostics (verbose = no)

(mode = ql)

-8-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

model_image

An  image  file of the fitted model is created.  This image may then

be displayed or a residual image may be calculated and displayed.

Multi-Spectra image  image =

Model image  model =

(mode = ql)

.nf

.le

.ls model_list

For the specified lines and spectra the model is listed.

The listing gives, for each spectra,

the spectrum number, the line number, the fitted position,

the estimated wavelength, the

extracted luminosity, the intensity scale, model width parameters, and

the background polynomial coefficients.  This routine can be used in scripts

to plot the extracted spectra, the trend of width with wavelength, and so

forth.

.nf

Multi-Spectra image  image =

Lines to list (lines = 1-1000)

Spectra to list (spectra = 1-1000)

(mode = ql)

sigma_extract

A previously fitted model is used to extract the spectra  luminosity

within  a  specified  sigma  of  the peak.  Because the model is not

necessarily a Gaussian the sigma is used to  compute  the  intensity

ratio  of  the  cutoff to the peak assumining a Gaussian profile and

then the data is extracted to the point the  model  intensity  falls

below  that  cutoff.   If  the  clean  option  is specified then the

model is used to detect pixels which deviate from  the  model  by  a

specified  amount.   The  model  value replaces the deviant pixel in

the extraction and, if specified, also in the image file.

Multi-Spectra image  image =

Sigma extraction width  width = 1.

Lines to extract (lines = 1-1000)

Clean spectra (clean = yes)

Cleaning threshold (threshold = 5)

Modify image (modify = yes)

Print general diagnostics (verbose = no)

(mode = ql)

-9-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

strip_extract

A strip of constant width about the spectra positions is  extracted.

If  cleanning  is  desired  a  smoothed  estimate  of the profile is

obtained by averaging a  number  of  lines  about  the  line  to  be

cleaned.   After  fitting  for  the intensity scale pixels are found

which deviate from the profile by a specified amount.   The  profile

value   replaces  the  deviant  pixel  in  the  extraction  and,  if

specified, also in the image file.  No prior  modeling  is  required

to use this extraction routine.

Multi-Spectra image  image =

Strip extraction width  width = 1.

Lines to extract (lines = 1-1000)

Clean spectra (clean = yes)

Cleaning threshold (threshold = 5)

Lines per profile average (averge_lines = 32)

Modify image (modify = yes)

Print general diagnostics (verbose = no)

(mode = ql)

to_iids

For  a  specified prefix, files of the form prefix.nn, where nn is a

specified spectra  number,  are  created  containing  the  extracted

spectra  for  all  the  specified  image  files.   The format of the

files is the IIDS format  developed  for  the  CYBER  Multi-Aperture

Plate Extractions.

Multi-Spectra image  images =

IIDS filename prefix  iids_file =

Spectra to format (spectra = 1-1000)

(mode = ql)

to_image

An  image  file  containing  one  line of the extracted luminosities

for each specified spectra in the specified MULTISPEC image.

Multi-Spectra image  in_image =

Extracted spectra image  out_image =

Spectra (spectra = 1-1000)

(mode = ql)

to_onedspec

The extractions are converted to an as yet to  be  specified  format

for use in the ONEDSPEC reduction package.

Multi-Spectra images  images =

ONEDSPEC data file  onedspec_file =

Spectra (spectra = 1-1000)

(mode = ql)

-10-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

4.2  General MULTISPEC CL Scripts

The  general  MULTISPEC CL scripts perform a series of steps needed

to extract the spectra from a specified  list  of  image  files.   These

steps  have  been found to generally perform the desired extraction task

fully.

multiap_extract         echelle_extract

multiap_extract

The specified multi-aperture plate  images  are  extracted.   If  no

starting  solution  image,  one which has previously been extracted,

is specified then the script performs an automatic  search  for  the

specified  number  of  spectra.   Otherwise  the  solution  from the

starting image is used as the  initial  model.   The  background  is

then  determined.   This is followed by a series of fitting steps on

swaths of data.  (For further details on the fitting steps  see  the

Algorithms   paper).    A   MODEL_EXTRACT   and   cleaning  follows.

Finally, the extraction is correlated with  the  specified  aperture

plate  using  AP_PLATE.   If  there  was no starting image then this

extraction becomes the initial solution  image.   Subsequent  images

are extracted starting from the initial solution image.

Multi-Aperture images  images =

Initial solution image  initial =

Aperture plate number  plate =

Number of spectra  nspectra =

(mode = ql)

echelle_extract

The   specified  echelle  images  are  extracted.   If  no  starting

solution  image,  one  which  has  previously  been  extracted,   is

specified  then  the  script  performs  an  automatic search for the

specified  number  of  orders.   Otherwise  the  solution  from  the

starting   image  is  used  as  the  initial  starting  point.   The

background  is  then  determined.   Finally  a   STRIP_EXTRACT   and

cleaning  is  performed.   If  there was no starting image then this

extraction becomes the initial solution  image.   Subsequent  images

are extracted starting from the initial solution image.

Echelle images  images =

Initial solution image  initial =

Number of orders  norders =

Extraction width  width =

(mode = ql)

5.  Outline of a MULTISPEC Reduction

The  following  outline  is for the reduction of a cryogenic camera

multi-aperture plate.  All the programmer supplied  default  values  are

used.

-11-

MULTISPEC (Oct83)  Multi-Spectra Extraction Package    MULTISPEC (Oct83)

(1)  rcamera mtb, "ap165.", "s", "3-9"

(2)  debias "ap165.*"

(3)  multispec_flat "ap165.[36]", "ap165.flat"

(4)  flat_divide "ap165.*", "ap165.flat"

(5)  multiap_extract "ap165.*", "", 165, 50

(6)  to_onedspec "ap165.*", oned165

(1) The  data  is  read  from  the observing tape(s) using RCAMERA.  The

image files created are ap165.3, ap165.4,  ...,  ap165.9.   This  is

easily  accomplished  by  using  the filename prefix "ap165." in the

RCAMERA program.  The raw images may be examined at  this  point  on

a display.

(2) The  images  are  debiased  using DEBIAS with all the "ap165." files

specified.  The debias program  knows  about  the  location  of  the

bias strip for the cryogenic camera.

(3) A  a  flat  field  is  created  using  MULTISPEC_FLAT  in  which the

desired  quartz  frames  are  specified  and  a  flat  field   image

filename  is  defined.  The created flat field image may be examined

on an image display if desired.

(4) All the  debiased  images  are  divided  by  the  flat  field  using

FLAT_DIVIDE.

(5) The  script  MULTIAP_EXTRACT  is  run  in  which  the aperture plate

number, the number of spectra, and the image files to  be  extracted

are  specified.   The  number  of  spectra  is found by examining an

image on an image display or by plotting a cut  across  the  spectra

using a general image profile program.

(6) Finally,  the  extracted  spectra  are  formatted  for  the ONEDSPEC

package using TO_ONEDSPEC with the extracted images specified.