Calculate the normal gravity on an ellipsoid or sphere using geocentric coordinates.

Usage

-----

value = NormalGravity (geocentriclat, gm, omega, a, b)

Returns

-------

value : float, ndarray

The normal gravity in SI units.

Parameters

----------

geocentriclat: float, array_like

Geocentric latitude in degrees.

gm : float

The gravitational constant multiplied by the mass of the planet.

omega : float

The angular rotation rate of the planet.

a : float

The semi-major axis of the flattened ellipsoid on which the normal gravity

is computed.

b : float

The semi-minor axis of the flattened ellipsoid on which the normal gravity

is computed.

Description

-----------

NormalGravity will compute the magnitude of the total gravity (gravitation and

centrifugal) on the surface of a rotating ellipsoid (in m/s^2).  The latitude

is input in geocentric coordinates in degrees.

For a rotating ellipsoid, the surface corresponds to a constant potential, and

the gravity vector is normal to the surface. The normal gravity is computed

using Somigliana's formula, as taken from Physical Geodesy (Hofmann-Wellenhof

and Moritz, 2nd ed., sections 2.7 and 2.8). In this routine, the geodetic

latitude is computed internally from the input geocentric latitude.

For the case of a sphere (a is equal to b), the normal gravity is defined as

the magnitude of the sum of the normal gravitation (GM/r^2) and the centrifugal

gravity. For a rotating sphere, the surface does not correspond to a constant

potential and the gravity vector is not normal to the surface.

References

----------

Hofmann-Wellenhof B, and H. Moritz, "Physical Geodesy," second edition,

Springer, Wien, 403 pp., 2006.